RO notes on data used in Figure 3 of the BirTomo paper

Ground truth object:

ShellSimulGT1.h5 -> for super sampling = 1, # of microlenses 23

object box {Z,Y,X}={5, 33, 33}, birefringence voxel values are negative between 0.0 and -0.01, optic axis values are normal to shell surface; the Z-components of the optic axis vectors are all either zero or positive.

Forward images of GT object simulated with BirTomo and 23 by 23 microlens array:

ShellSimulGT1LFImage_Ret.tif, ShellSimulGT1LFImage_Azim.tif

The above light field images have zero retardance in one row and column of microlenses around the edges of the LF image. The retardance is actually not zero but consistently 2.98•10-8. The associated azimuth is zero.

Initial random volume for starting the reconstruction:

ShellInitRandom1.h5 -> Initial volume with random birefringence (-0.01<∆n<0) and optic axis values in central object box {3, 23, 23}, which was padded by {1, 5, 5} on each side, leading to object box {Z,Y,X}={5, 33, 33}.

For the forward projection and the iterative reconstruction, the following parameter values were used in the file optical_config.json

{

"volume_shape"      : [5,33,33],

"n_micro_lenses"    : 23,

"pixels_per_ml"     : 16,

"n_voxels_per_ml"   : 1,

"M_obj"             : 60,

"na_obj"            : 1.4,

"n_medium"          : 1.52,

"wavelength"        : 0.482,

"camera_pix_pitch"  : 7.4,

"aperture_radius_px": 8,

"cube_voxels"       : true,

"axial_voxel_size_um"     : 2.0,

"polarizer"         : [[1, 0], [0, 1]],

"analyzer"          : [[1, 0], [0, 1]],

"polarizer_swing"   : 0.156,

"precision"         : {

"jones": "float64",

"ret_azim_final": "float32",

"intensity": "float32",

"clamp_eps": true

}

}

For the iterative reconstruction, the following parameter values were used in the file iter_config.json

{

"general": {

"num_iterations": 500,

"save_freq": 251,

"output_directory_postfix": " ShellSimPap1_ShellInitRandom1",

"notes": ""

},

"learning_rates": {

"birefringence": 0.0001,

"optic_axis": 0.1

},

"regularization": {

"weight": 0.5,

"functions": [

["birefringence active L2", 100],

["birefringence active negative penalty", 0]

]

},

"file_paths": {

"initial_volume": null,

"saved_rays": null,

"vox_indices_by_mla_idx": null,

"ret_image": null,

"azim_image": null,

"radiometry": null

},

"schedulers": {

"birefringence": {

"type": "ReduceLROnPlateau",

"params": {

"mode": "min",

"factor": 0.8,

"patience": 5,

"threshold": 1e-6,

"min_lr": 1e-8

}

},

"optic_axis": {

"type": "CosineAnnealingWarmRestarts",

"params": {

"T_0": 100,

"T_mult": 2,

"eta_min": 1e-4

}

}

},

"nerf": {

"enabled": false,

…

}

}