RO notes on data used in Figure 4 of the BirTomo paper

Experimental light field images:

XylemACrop4Float32bitRet.tif, XylemACrop4Float32bitAzim.tif,

Initial random volume:

XylemACrop4Random.h5 -> XylemACrop4Random.h5, object box {Z,Y,X}={25, 121, 121}, random birefringence pixel values are positive between 0.0 and 0.01, random optic axis values; the Z-components of the optic axis vectors are all either zero or positive; random values are in central object box {25, 81, 81}, which was padded by {0, 20, 20} on each side, leading to object box {Z,Y,X}={25, 121, 121}.

For the forward projection and the iterative reconstruction, the following parameter values were used in the file optical_config.json

{

"volume_shape":    [25, 121, 121],

"n_micro_lenses"    : 61,

"pixels_per_ml"     : 16,

"n_voxels_per_ml"   : 1,

"M_obj"             : 60,

"na_obj"            : 1.2,

"n_medium"          : 1.52,

"wavelength"        : 0.544,

"camera_pix_pitch"  : 6.5,

"aperture_radius_px": 8.0,

"cube_voxels"       : true,

"axial_voxel_size_um"     : 1.73,

"precision"         : {

"jones": "float64",

"ret_azim_final": "float32",

"intensity": "float32",

"clamp_eps": true

}

}

For the iterative reconstruction, the following parameter values were used in the file iter_config.json

{

"general": {

"num_iterations": 500,

"save_freq": 251,

"output_directory_postfix": " XylemACrop4_XylemACrop4Random",

"notes": ""

},

"learning_rates": {

"birefringence": 0.0001,

"optic_axis": 0.1

},

"regularization": {

"weight": 0.5,

"functions": [

["birefringence active L2", 100],

["birefringence active negative penalty", 0]

]

},

"file_paths": {

"initial_volume": null,

"saved_rays": null,

"vox_indices_by_mla_idx": null,

"ret_image": null,

"azim_image": null,

"radiometry": null

},

"schedulers": {

"birefringence": {

"type": "ReduceLROnPlateau",

"params": {

"mode": "min",

"factor": 0.8,

"patience": 5,

"threshold": 1e-6,

"min_lr": 1e-8

}

},

"optic_axis": {

"type": "CosineAnnealingWarmRestarts",

"params": {

"T_0": 100,

"T_mult": 2,

"eta_min": 1e-4

}

}

},

"nerf": {

"enabled": false,

…

}

}